A Manager/Worker POMDP Decomposition of the G1 Meta-Reasoning Stack

Adrián Lendínez Ibáñez — working note in response to RQ, 21 July 2026. Goal: the list of high-level and low-level states, observations and actions we agreed to draft, with each element mapped to its already-implemented counterpart in the G1 stack and to the logged fields that would ground the particle-filter probabilities. Everything in code font is a field recorded per run today.

1. The framing in one paragraph

Low-level policies are the analogies — each a capability-scoped skill (multi-parameter profile {v_cap, turn_cap, robot_r} + engagement subgoals) documented in the Capability Map. The Manager POMDP is the meta-reasoner: it holds beliefs over regime variables it cannot observe directly (payload state, environment regime, sensing health, analogy validity, progress), and its actions set the context the Worker runs in — which analogy, which intermediate subgoal, which parameter bounds, which sensor to trust, when to explore, when to stop and ask. The Worker POMDP is the per-analogy executor: beliefs over the local obstacle field and own pose, actions = trajectories and recovery primitives. The Worker runs as an option (semi-MDP) until the Manager changes context — RQ’s Manager/Worker split is exactly this options architecture.

2. Manager POMDP (meta-reasoner, 0.5–2 Hz)

2a. Hidden state variables and their observable proxies

| Hidden state (belief over…) | Observable proxies (logged) | Current belief machinery |
| --- | --- | --- |
| Payload state: lid open/closed, fill level, settled vs fresh cup | spill marks (spills_human, UDP GT), spill rate, time since last spill | Fragility channel: exponential dips + repeat load (platform-calibrated, G1_M2_REALCAL) |
| Environment regime: open corridor / door zone / tight pocket | c0, c0_hard, clearL_m/clearR_m, median clearance, door distance | Layer 3 relevance EMA (meta2_env, clamp 0.85–1.15) |
| Sensing health: laser trustworthy? camera needed? localization ok? | laser_noise, filt_rej, scan_churn, loc_conf, loc_match, reloc_rate10s, color_near, perc_age | SensingMonitor reliability -> per-reading DST uncertainty (meta2_unc) |
| Platform resources: battery sufficient? thermal/mechanical health? | bat, cpuT, per-joint telemetry (1 Hz track), IMU | Battery reading into reasoner; no belief dynamics yet (gap) |
| Analogy validity: is the active low-level policy still matched? | fulfilment meta2_ful, tension meta2_tens, spills under this analogy, QoE region occupancy | Layer 2 DST per analogy {m_match, m_mismatch, Θ}; Pl thresholds 0.45/0.70 |
| Progress state: advancing / stuck / looping (a STATE, as Adrián noted — the action it licenses is exploration or escalation) | progress rate, path/goal distance history, collision events, P9 windows | Escalation windows (G1_M2_ABORT_*); aggressive-mode trigger |
| Decision robustness: how safe is the current arbitration itself? | top-2 fulfilment margin, uncertainty gap, attention mismatch | ρ_DCA per decision (meta2_rho) |

2b. High-level ACTIONS (the Manager list)

| # | Manager action | Implemented as | Status |
| --- | --- | --- | --- |
| M1 | Select / switch low-level policy (analogy) | KEEP/SWITCH/FALLBACK over Cautious/Efficient with DST deploy veto | DONE (twin + real) |
| M2 | Set intermediate subgoal for the analogy | Door-approach variant (Direct 0.85 m / Far 1.20 m / BiasPlus +0.12 m) parametrises the engagement FSM | DONE at the door; generalisation = next research line (B-pocket entry pose) |
| M3 | Bound the Worker (parameter governance) | Profile caps {v, turn, robot_r}; L3 modulates with environment relevance; graded spill->speed coupling | DONE |
| M4 | Arbitrate sensors (lidar vs camera) | COLOR-BRAKE: camera moderates when laser is height-band blind; yaw-gate: distrust laser while turning; DOORSTICKY: trust static map over flickering returns near the door | DONE as reflex rules; formalising as belief-conditioned action is the PF opportunity |
| M5 | Explore vs exploit | G1_M2_DOOR_EXPLORE: least-tried variant forced in the TWIN (failure is free); hardware always exploits by Pl | DONE today — results in §4 |
| M6 | Reuse / seed skills | Ext C sim-to-real trust transfer (G1_M2_STATE_INIT, task-id remap); L4 start-up re-selection | DONE |
| M7 | Escalate to human (HELP) / governed stop | P9 escalation; HELP sustained -> stop (the 23 s governed stop vs 513 s ungoverned loop pair) | DONE |
| M8 | Refuse / renegotiate task on resources | Battery observed but no return-to-base / refusal policy | GAP (candidate M-action) |

3. Worker POMDP (per-analogy executor, 3–10 Hz)

3a. Hidden state and observations

| Hidden state (belief over…) | Observable proxies (logged) | Current belief machinery |
| --- | --- | --- |
| Local obstacle field — including what the laser cannot see (below 0.7 m band; thin jambs edge-on) | laser cells per scan (laser_snapshots now with pose+motion phase), camera clamp columns, color_rmin, IMU contact | K-of-N persistence filter + per-cell score (oscore) = a discrete occupancy filter; DOORSTICKY freezes known-static cells |
| Own pose quality | SLAM pose, loc_conf, loc_match, reloc jumps | Gate on start (GATE_M); jump detection; no full pose posterior (PF opportunity) |
| Trajectory feasibility ("is a feasible trajectory?") | DWA candidate scoring vs clearance, plan_n (A* success), carrot errors | DWA + A* replan (the MDP solver RQ names) |
| Contact risk of own envelope | c0 vs governed robot_r, clearL/R balance | Governed body radius (was the mechanism-A failure when fixed at 0.22) |

3b. Low-level ACTIONS (the Worker list)

| # | Worker action | Implemented as |
| --- | --- | --- |
| W1 | Trajectory selection within bounds | DWA velocity commands under profile caps |
| W2 | Global replan | A* on static+confirmed costmap |
| W3 | Engagement primitives | Door FSM: align -> strafe -> cross (subgoal set by M2) |
| W4 | Recovery primitives | Back-off, rotate, ESCAPE (never governed — safety floor) |
| W5 | Reflex guards | HARD-GUARD stop/slow on persistent walls; COLOR-BRAKE creep/stop (camera-only); VCAP |
| W6 | Epistemic actions on its own map | Insert/decay cells, yaw-gate freeze while turning, DOORSTICKY fixation — actions taken to manage belief, not motion |

4. Exploration as a Manager action: first result (digital twin)

12-run twin arm with forced least-tried rotation: 4 trials per variant (vs 8/0/0 under pure exploitation in the formal A/B). All three variants now carry grounded trust (Θ shrank from 0.50 to 0.12–0.16). Door_Direct confirmed as incumbent with full evidence (4/4 crossings, 0 engagement fails); Door_Far and Door_BiasPlus each took one engagement fail; BiasPlus additionally suffered two post-door pocket aborts — correctly charged to the pocket, not to the door phase (credit assignment respects the control envelope, the July-14 lesson). The ranking cost three aborted twin runs — free — and ships as meta2_state_twin_explored.json, the recommended G1_M2_STATE_INIT seed for real campaign 2.

| Variant | Trials | Crossings | Fails | Final Pl | Mean reached time (s) |
| --- | --- | --- | --- | --- | --- |
| Door_Direct | 4 | 4 | 0 | 1.00 | 93.6 |
| Door_Far | 4 | 3 | 1 | 0.93 | 103.0 |
| Door_BiasPlus | 4 | 3 | 1 | 0.93 | 107.4 (1/4 runs reached: 2 post-door pocket aborts) |

The populated library seeds the real robot through Ext C (G1_M2_STATE_INIT): exploration happens where failure is free; the hardware only ever exploits. The resulting state file ships with the repo for campaign 2.

5. Where the particle filter fits — and what already grounds it

•  Correspondence. The DST masses are already a belief: Bel = m_match is the pessimistic posterior, Pl = m_match + Θ the optimistic one, and Dempster’s rule the evidence update. A particle filter adds what DST lacks: TEMPORAL dynamics (transition models for regime variables — cup settling, battery drain, sensing degradation while turning) and joint posteriors over correlated hidden states.

•  Worker side. The K-of-N persistence filter + cell scores are a per-cell binary Bayes filter; a PF over (pose, local map) would unify it with localization quality — RQ’s “Particle Filters and MDP solver” is literally this plus the existing DWA/A*.

•  Calibration corpus already logged. Every observable named in §2–3 is recorded per run: 140+ twin runs and 60+ real runs, including per-sample DST uncertainties (meta2_unc), laser snapshots with pose+motion phase (for the emission model, noise +65% while turning), spill ground truth, and per-run outcomes. Defining the transition/emission models and fitting them to this corpus is a data-analysis task, not a new experimental campaign.

Proposed next step: we draft the formal tuple ⟨S, A, O, T, R⟩ for the Manager with 5–6 discrete regime variables (payload, environment, sensing, resource, validity, progress), reward = the existing meta-attention weights (progression/safety/fragility/battery/mobility — they are already the reward vector in the configs), and fit T/O from the logged campaigns; the Worker keeps its current solvers with the PF formalisation of its filters. That yields RQ’s “complete framework” with the implemented system as its reference implementation, and the thesis/paper narrative: the framework was not designed on paper and then built — it was distilled from a working, measured stack.

6. State estimation without RL: the particle filter now runs in shadow mode

Per RQ’s point (21 Jul, 10:59–11:00): transition probabilities are unknown, so optimising the MDP classically would demand RL — instead, the decision policy stays inside the QoE-analogy pipeline and the particle filter is used ONLY as the Manager’s state estimator. Implemented today (g1_particle_filter.py, bridged behind G1_M2_PF, shadow mode: posteriors are computed and logged per sample as meta2_pf, no decision path is touched): 400 particles over a hybrid state — lid ∈ {open, closed}, fill ∈ [0,1] (continuous — the reason this is a PF and not an exact HMM), sensing health, battery sufficiency, stuck. Observation models calibrated from the logged campaigns (spill rate vs motion excitation and freeboard; laser noise +65% while turning).

Replay validation over logged runs (no new experiments needed):

| Run (logged) | Ground truth | P(lid open) final | Fill estimate |
| --- | --- | --- | --- |
| Real baseline 102149 (14 spills) | open, 246 g full | 1.00 | 0.90 → 0.42 (14 spills ≈ 0.033 each — matches 7 g/spill / 200 g measured) |
| Real governed 171700 (12 marks) | open, 150 g | 1.00 | 0.90 → 0.45 declining with marks |
| Real R2 abort 164422 (4 marks in 8 s) | open, fresh cup | 1.00 by t≈8 s | front-loaded drop |
| Twin m2closedsim 125800 (0 spills, full motion) | CLOSED | 0.00 | 0.91 (nothing spilled — correct) |
| Twin m2closedsim 130149 (0 spills) | CLOSED | 0.00 | 0.90 |

RQ’s “is the lid open or closed” is now an inferred belief, not an operator setting — discriminated correctly on all five replayed runs. Unified-pipeline hook (v2, after validating against the pending sealed-lid real runs R4): condition QoE expectations on the posterior — e.g. P(open) < 0.2 activates the covered-delivery analogy set — so the belief feeds the existing arbitration and the policy still never needs RL.

6a. Estimator per evidence density (RQ, 11:07–11:10) — settled by data, not argument

RQ’s refinement: particle filters shine where samples are dense (navigation — laser returns, pose hypotheses), and are weak for sparse events (lid, battery); “no matter DST or particle filter or others, they are all probabilistic input, all feeding the decision model under POMDP.” The stack now embodies exactly that: on the Worker side the K-of-N persistence filter + cell scores already are a sampled-evidence occupancy filter over dense returns; on the Manager side the 3-layer DST handles sparse events with explicit ignorance Θ. One empirical nuance worth adding to the discussion: for the lid, the ABSENCE of a spill under motion excitation is itself a Bernoulli observation at 2 Hz — dense evidence — which is why the shadow PF converged to P(open)=0.00 on the closed-lid twin runs from ZERO events. And because the PF runs in shadow while the DST layers run live, both estimators now execute in parallel on every run over identical inputs: Adrián’s proposal (“we will try particle filter as the estimator, if it doesn’t work… maybe the 3-layer DST is enough”) becomes a measurable comparison on logged data rather than a design debate.