G1 Physical-Robot Payload Campaign — Operator Guide

Twenty runs, four conditions, manual spill ground truth, cross-run belief state — everything needed to produce the env=real results for Section VII.H, mirroring the digital-twin campaign

Unitree G1 “Air” · real laboratory · July 2026 · companion to “G1 Digital-Twin Payload Experiment” (rev. 4)

1.  What this campaign must produce

- The env=real rows of Table 6: time, spills/run (human-marked), collisions and completion per condition — directly comparable with the twin because metrics, dataset schema and analysis scripts are identical.

- The Layer-2 recovery curve on real hardware (the one result the twin could not produce): under the wrong-prior condition the fast analogy should genuinely cruise in the open lab (clearance ≈ 1.0 certifies it), gait-induced spills should occur, the persisted plausibility should erode run by run, and a later run should START pre-corrected. This is thesis Fig. 5.3.6.2-3 with physics.

- Live ρ̂ traces on hardware (meta2_rho is logged automatically) for the certification-ratio figure and the future B-calibration.

Every run records automatically: per-tick samples (pose, clearance, progression, mobility, META2 action/analogy/tension/fulfilment/Pl/ρ̂/env), events (collisions, spills, governance transitions), telemetry, and camera frames. You only add: the spill marks, and one line of condition notes per run in runs_summary.csv afterwards.

2.  Setup (once per session)

2.1  Hardware checklist

- Robot charged > 80%, standing in walk mode, relocalized in the vendor app (SLAM screen, map loaded, camera ON).

- iPhone connected by USB to the Ubuntu machine, trusted; Web Inspector ON; do NOT open Safari's inspector on that page.

- Physical remote in hand at all times — L2+B is the kill switch (damping stop).

- 2-3 m of clear space around the route; water only, never coffee; floor dry.

- The cup: same cup for the whole campaign, radius ≈ 4 cm, filled to 1 cm below the rim (matches the model freeboard). Refill to the same level before every run. Lid available for the sealed condition.

2.2  Software bring-up (Ubuntu, 3 terminals)

Terminal 0 — always first:

cd ~/G1_UNITREE_ROBOT_META_REASONING && git pull

ios_webkit_debug_proxy

Terminal 1 — perception server (required: the perception gate blocks navigation without verified vision):

source venv/bin/activate

G1_FLOORCOLOR=1 python perception_server.py --host 0.0.0.0 --port 8008 \

--fx 300 --fy 300 --cx 160 --cy 120 --cam-h 1.10 --cam-pitch -10

Terminal 2 — the run (arm-specific command, §4). Confirm at start-up: `[perc] ... -> OK`, `PERC-TEST ... floorcolor=ON`, and the governance banner (`META2 ACTIVO ... analogia inicial ...`) when governed.

Terminal 3 — the spill marker (§3). Leave it running for the whole session.

3.  Spill ground truth: the technician's marker

One person watches the cup for the entire run and marks every spill event — any amount of water leaving the cup counts as one event; a continuous slosh-over is one event, a second distinct slosh is a second event. When in doubt, mark it and write a note. The mark is timestamped into the run dataset (kind = spill_human) and counted into the summary (spills_human); it also feeds the governance fragility channel in real time, so the robot itself reacts to marked spills (one spill → caution; two → FALLBACK ceiling; three or more → HELP stop: that is correct behaviour, not a malfunction).

3.1  How to mark (physical robot — UDP channel)

The physical stack is ROS-free (it drives the robot through the vendor app's WebRTC session), so the marker is a UDP datagram to the listener that every run opens on port 7777. Three equivalent ways:

A. The marker tool (recommended) — press ENTER once per spill:

python spill_mark.py                 # on the Ubuntu machine, or

python spill_mark.py <UBUNTU_IP>     # from any laptop on the same LAN

B. One-liner (any machine with netcat):

echo spill | nc -u -w1 <UBUNTU_IP> 7777

C. ROS bridge (only if you are running the simulator, or a ROS 2 laptop with the forwarder) — in simulation this topic is forwarded automatically to the same UDP channel:

ros2 topic pub --once /spill_event std_msgs/msg/Empty

Verification before run 1 of the session: send one test mark while a throwaway run is active and confirm the Terminal-2 line `[spill-GT] DERRAME humano #1 ...` appears. Abort that throwaway run (Ctrl+C) — it saves a dataset; note it as a test in the CSV. If no line appears: check the IP, and that port 7777/UDP is not firewalled.

4.  The campaign: 20 runs, four conditions

Alternate direction every run within an arm (A→B, then B→A — the robot is already at the far waypoint), and note the direction. Run the arms IN THIS ORDER (the wrong-prior arm must come after the operator is fluent with the procedure):

| # | Arm (runs) | Cup | Purpose |
| --- | --- | --- | --- |
| R1 | R-M0 baseline — no governance (4) | open | The ungoverned reference: fast, expected to spill in cruise. Comparable to twin M0. |
| R2 | R-M2 payload governed (6) | open | The proposed system: full four-layer configuration, persisted state. Expected ~0 spills at a bounded time cost. |
| R3 | R-WRONG prior + Layer 2 (6) | open | THE key arm: door-task (Efficient-preferred) configuration with an open cup — deliberately mis-calibrated. In the real lab the fast analogy certifies in open floor: expect early-run spills, plausibility erosion in the state file after each run, and a later run starting pre-corrected in Cautious. |
| R4 | Sealed lid: 2 × M0 + 2 × M2-covered (4) | sealed | Lid-state delta on hardware: passive physics for M0, task-conditioned exploitation for the governed covered-delivery configuration. |

Table 1. Campaign design (20 runs). If time allows, extend R2 and R3 first — they carry the paper.

4.1  Exact commands per arm (Terminal 2)

R1 — baseline (4 runs):

G1_META2=0 G1_PERC=127.0.0.1:8008 python g1_goto.py gotoviz B   # then A, B, A

R2 — governed payload (6 runs). Delete the state file ONCE before run 1, never between runs:

rm -f meta2_state_real_m2.json        # ONLY before run 1 of the arm

G1_META2=2 G1_M2_CFG=config_meta2_g1payload.json \

G1_M2_STATE=meta2_state_real_m2.json G1_M2_L4=1 G1_M2_L3=1 \

G1_PERC=127.0.0.1:8008 python g1_goto.py gotoviz B    # alternate B / A

R3 — wrong prior + Layer 2 (6 runs). Fresh state before run 1; keep it afterwards:

rm -f meta2_state_real_wrong.json     # ONLY before run 1 of the arm

G1_META2=2 G1_M2_CFG=config_meta2_g1door.json \

G1_M2_STATE=meta2_state_real_wrong.json \

G1_PERC=127.0.0.1:8008 python g1_goto.py gotoviz B    # alternate B / A

R4 — sealed lid (4 runs; put the lid on the cup):

G1_META2=0 G1_PERC=127.0.0.1:8008 python g1_goto.py gotoviz B          # x2 M0

G1_META2=2 G1_M2_CFG=config_meta2_g1door.json \

G1_M2_STATE=meta2_state_real_lid.json \

G1_PERC=127.0.0.1:8008 python g1_goto.py gotoviz B                      # x2 covered

Notes: L3 in R3/R4 is optional (leave it off to keep the wrong-prior dynamics clean; R2 carries the L3 evidence). Do NOT set the simulation escalation windows — the real robot uses the validated defaults. All other safety layers (relocalisation guard, hard-guard, door engagement, escape) are ON by default; leave them.

5.  Per-run checklist

Before

- Cup refilled to the mark (1 cm below rim); lid per condition; cup handed to the robot / mounted as in previous sessions.

- Robot at the start waypoint, relocalized (pose sane in the app), battery noted.

- Marker terminal alive; observer in position with line of sight to the cup for the whole route.

- Correct arm command with the right G1_M2_STATE file (never mix state files across arms).

During

- Mark every spill immediately (one press per event).

- If the robot stops and the log shows sustained HELP / meta2_experience_abort: that is a governed abort — a valid result. Do not restart the run; record it.

- Kill switch only for genuine safety (a fall risk, a person entering the space). Note any manual intervention.

After

- Terminal 2 prints the dataset path (`[dataset] travesia guardada -> ...`). Confirm `spills_human` appears in the summary if you marked any.

- For R3: inspect the state file after EACH run and write down the plausibilities:

python -m json.tool meta2_state_real_wrong.json | grep -A3 m_match

- Every 2-3 runs: `git add dataset/ *.json && git commit -m "real runs ..." && git push origin HEAD:main` (resolve non-fast-forward with `git pull --no-rebase` first).

6.  What the theory predicts (check as you go)

| Arm | Expected signature | If instead you see... |
| --- | --- | --- |
| R-M0 | Spills in open-floor cruise (gait slosh at 0.30 m/s), few/none at the door (slow there anyway); time ≈ 55-90 s per leg | No spills at all in 4 runs → cup fuller / marker definition — verify fill level and continue; the contrast arm is what matters |
| R-M2 | ≈ 0 spills; visibly slower cruise (0.18 m/s ceiling, relaxed by L3 in open floor); FALLBACK lines near the door are normal | Repeated HELP aborts far from any obstacle → note run IDs; likely fragility over-marking or a QoE issue — finish the arm, we analyse offline |
| R-WRONG | Early runs: fast cruise + spills; state file Pl(Efficient) dropping after each spill run (1.0 → ~0.75 → ~0.5 ...); once Pl < 0.70 the ceiling blends down mid-arm; once Pl < 0.45 a run STARTS in Cautious — write down WHICH run that is | No spills at all in runs 1-2 → the real cruise may be gentler than modelled: extend this arm before any other, and check the marker line appears on a deliberate test mark |
| R4 lid | M0: fewer spills, same speed. M2-covered: faster than R-M2 at ≈ 0 spills | — |

The one plot we are after: Pl(Efficient_Nav) per completed R3 run, from your noted values — the hardware version of Figure 5. If it emerges, the analogical-trust story is closed end-to-end.

7.  Troubleshooting

| Symptom | Cause / action |
| --- | --- |
| PERC-GATE BLOCKED at start | Perception server not up or camera off: check Terminal 1, app camera ON. (G1_NOVIS=1 bypasses — baseline arm only, never governed payload runs.) |
| No [spill-GT] line on a test mark | Wrong IP / UDP 7777 firewalled / marker aimed at localhost from another machine — use the Ubuntu IP. |
| RELOCALIZACION DUDOSA / robot will not start | Re-localize in the app at the waypoint; check the app map matches reality. |
| Robot stops mid-run, log shows META2 ... HELP | Governed abort — a RESULT. Record it, reset the robot to a waypoint, continue with the next run. |
| Repeated reloc jumps / STOP by guard | Known guard (G1_RELOCGUARD). Re-localize; if recurrent, note and continue — the dataset is still valid up to the stop. |
| git push rejected (non-fast-forward) | git pull --no-rebase, then push origin HEAD:main. |
| Wrong state file used for an arm | Note it immediately; that run moves to the other arm's count or is discarded — do not delete anything, the audit sorts it out. |

8.  After the campaign: hand the data back

- Final `git push` of all datasets + the three `meta2_state_real_*.json` files (they are data: they contain the trust trajectories).

- Fill `condition` and `notes` per run in runs_summary.csv (`python summarize_runs.py` regenerates it; runs are tagged env=real automatically).

- Your handwritten Pl values per R3 run (photo of the notebook is fine).

Analysis then mirrors the twin campaign automatically: Table 6 gains its env=real rows, Figure 5 gains the hardware trace, ρ̂ gets its B-calibration data, and the sim↔real closing paragraph of Section VII.H writes itself from the comparison.

Companion documents: “G1 Digital-Twin Payload Experiment” rev. 4 (methods & theory), HANDOFF.md §8.18 (flags reference), COMO_LANZAR.md (simulator). All versioned with the stack.