G1 Physical Campaign — Interim Report (Session 1)

2026-07-08, real laboratory · 8 runs (6 baseline + 2 governed) · session paused ~11:10, to be resumed

1.  Executive summary

The first hardware session produced the two most important single data points of the entire evaluation. (i) The ungoverned baseline reproduced the historical failure mode: run 6 wedged into a door-frame collision loop for 513 s (7 collisions, 50 m of thrashing) until manually killed. (ii) The governed system demonstrated the full HELP chain on hardware: with a brimful cup, real human spill marks drove the fragility meta-parameter into its dangerous region, the hard veto emptied the deployable set, sustained HELP triggered the experience-escalation layer, and the robot stopped itself in 23 s — a governed, bounded, self-diagnosed stop versus an 8.5-minute human-supervised failure. This is the paper's conservative non-selection semantics with physical ground truth. The practical consequence — a brimful cup (1 cm freeboard) cannot survive the start-manoeuvre turns under any speed governance — is itself a finding (turn rate is not yet part of the analogy profile) and sets the protocol for session 2 (200 g fill for completed governed runs).

2.  Runs completed

| Run | Arm / leg | Result | Time (s) | Spill marks | Collisions | Water |
| --- | --- | --- | --- | --- | --- | --- |
| 102149 | R1 baseline A→B | reached | 74.6 | 14 | 0 | 246 g fill |
| 102421 | R1 baseline B→A | reached | 48.4 | 3 | 0 | → 116 g (RT: 130 g) |
| 103218 | R1 baseline A→B | reached | 99.7 | 18 | 2 (IMU grazes) | 246 g fill |
| 103558 | R1 baseline B→A | reached | 124.3 | 8 | 0 | → 96 g (RT: 150 g) |
| 104244 | R1 baseline A→B | reached | 106.0 | 27 | 0 | 246 g fill |
| 104520 | R1 baseline B→A | MANUAL ABORT | 513 | 3 | 7 (door frame) | → 77 g (RT: 169 g) |
| 110018 | R2 governed A→B | HELP abort (governed) | 23.0 | 4 (t=10-14 s) | 0 | 246→162 g |
| 110554 | R2 governed A→B | HELP abort (governed) | 14.6 | 4 (t=3.5-6.7 s) | 0 | 246→209 g |

Baseline aggregate (6 runs): 5/6 reached, 12.2 spill marks/run, ~449 g of water lost in total (5.6-7.6 g per marked event), 9 collisions, reached-run times 48-124 s (mean ≈ 91 s). Fresh-cup front-loading (most spills in the first ~25 s) reproduced in 3 of 3 fresh fills.

3.  Real robot vs digital twin

| Quantity | Digital twin (82 runs) | Real robot (session 1) | Reading |
| --- | --- | --- | --- |
| Baseline time A→B | 83.4 ± 5.2 s | 48-124 s, mean ≈ 91 s | Consistent; real variance larger |
| Baseline spills/run | 0.50 ± 0.37 | 12.2 marks/run (~75 g/run) | Real gait ≈ 20× more excitatory than modelled: K_GAIT recalibration datum. Direction of the governed contrast unchanged |
| Water per spill event | FB_LOSS = 4 mm column (assumed) | 5.6-7.6 g ≈ 1.4-1.9 mm column | Model constant to be re-fit (≈ 1.5 mm) |
| Fresh-cup front-loading | Predicted (freeboard mechanism) | Reproduced 3/3 fresh fills | Mechanism validated on hardware |
| Baseline failure mode | No door loop in twin (no clutter, wider twin door) | 513 s door-frame loop, 7 collisions, manual kill | The historical failure (run 100927-style) is real and ungoverned; motivates escalation |
| HELP semantics | 3/8 single-candidate arm aborts (mobility veto) | 2/2 governed fresh-cup runs: spill marks → fragility veto → HELP → stop in 23.0 / 14.6 s | Full chain confirmed on hardware with human ground truth — first such demonstration |
| Start-phase spills | Present but below spill threshold (smooth planar start) | 3-4 marks in first 15 s of EVERY fresh-cup run, governed or not | NEW finding: start turns dominate; turn rate is not governed (only forward speed) — post-campaign work item: angular-speed in analogy profiles |
| Governed completed runs | 8/8 reached, 0 spills | None yet (2 HELP aborts at 246 g fill) | Session 2: 200 g fill (“payload level B”) + 2 baseline runs at 200 g for comparability |

4.  Theory validation status after session 1

Confirmed on hardware: (a) conservative non-selection / HELP semantics with real human ground truth feeding the fragility channel — the single most important governance claim; (b) the spill model's freeboard mechanism (front-loading) and jerk-dominance (start turns, not cruise speed alone); (c) the ungoverned failure mode that experience escalation exists to prevent, timed at 513 s versus 23 s governed. Pending (session 2): the headline availability-vs-deployability trade-off needs completed governed runs (200 g protocol); the Layer-2 cross-run recovery (wrong-prior arm R3) and the sealed-lid deltas (R4) are untouched. New knowledge produced by hardware: turn manoeuvres are the dominant start-phase spill source and are currently outside the governed profile — an extension (angular-rate ceilings per analogy, mirroring the thesis profiles' multi-parameter policies) is queued for after the campaign.

5.  Session 2 checklist

(1) Robot relocalised at a waypoint; perception + proxy + marker terminals up. (2) R2 remaining runs at 200 g fill, same command and state file (meta2_state_real_m2.json — REMEMBER to include it in git commits; it already holds two runs of mismatch evidence). (3) Then R3 wrong-prior (fresh meta2_state_real_wrong.json, 6 runs, note Pl after each). (4) R4 sealed lid (4 runs). (5) Two baseline runs at 200 g for the fill-level pairing. Marking discipline: a continuous slosh burst = ONE mark; ≥2 s calm separates events.

Data: all 8 runs pushed to main (datasets + filmstrips + goto.log). Companion documents: Digital-Twin Experiment rev. 4, Real-Robot Campaign Guide.